| 1: Identification of the article/product and of the company/ undertaking |  |
| --- | --- |
| 1.1: Product identifier |  |
| Article/Product name | Sintered hard metal articles containing primarily tungsten carbide with cobalt binder. [Add specific name if relevant] Synonyms: Cemented Carbide, Cemented WC, Cemented Tungsten Carbide. |

| 1.2: Relevant identified uses of the article and uses advised against |  |
| --- | --- |
| Identified uses | Mining Tools, Construction Tools, Round Tools, Metalworking Tools, Metallurgical Products, and Inserts. |
| Uses advised against | Hard metal articles and tools should not be reground or sharpened without taking appropriate safety measures to contain dust and to prevent exposure to dust (e.g. ventilation, personal protective equipment). Operations such as grinding, cutting, burning and welding of hard metal articles and tools may produce dust or fumes, which can be inhaled, swallowed or come in contact with the skin and eyes. |

| 1.3: Details of the supplier of the safety information sheet |  |
| --- | --- |
| Name | [Name of Company selling/supplying article] |
| Address | [Company-specific contact information] |
| Phone | [Company-specific contact information] |
| E-mail | [Company-specific contact information] |

| 1.4: Emergency telephone number |  |
| --- | --- |
| - Emergency telephone number | Not applicable |

2: Hazards identification

WARNING

Mechanical/Physical hazard: Hard metal articles and tools may fragment in use. Metal chips could be very hot with sharp edges and should not be touched by hand. Chips could cause burns to the skin and damage to the eyes.

Dust and mist hazard:	Operations such as grinding, cutting, burning and welding of hard metal articles may produce dust or fumes, which could be inhaled, swallowed or come in contact with the skin and eyes.

Sensitizing hazard:	Contact with uncoated solid hard metal may cause an allergic skin reaction as a result of prolonged skin contact. Cobalt are known sensitizers having potential to cause allergy through repeated exposure. A sensitized person could react with asthmatic symptoms or eczema.

HARD METAL, in particular, powder, dust and fumes, may present further health hazards as described in this safety information sheet. The health hazards described relate to hard metal and its individual components, such as tungsten carbide and cobalt.

| 2.1: Classification of the article |  |
| --- | --- |
| Classification according to 29 CFR 1910.1200(GHS), EC 1272/2008 (CLP): | Not applicable for articles |

| 2.2: Label elements |  |
| --- | --- |
| The following information can occur on the product warning label |  |
| Signal Word: | WARNING |
| Package label text | DUST AND MIST HAZARD! If grinding the product, use approved personal protective equipment as inhaling dust/mist could cause serious injury or death (permanent respiratory disease). FLYING FRAGMENT HAZARD! Product may shatter and could cause injury. Use approved personal protective equipment. DUST AND MIST HAZARD! Flying fragment hazard! Use approved personal protective equipment. DUST AND MIST HAZARD! Flying fragment hazard! Warning: Product may break or shatter in use. Grinding this product produces dust or mist that, at levels above the occupational exposure limit, could cause permanent respiratory disease, and could irritate nose, throat, skin and eyes. Contains one or more of: tungsten carbide, cobalt. Read Safety Data Sheet, Safety Information Sheet or contact your sales representative. To avoid injury, use personal protective equipment, including eye protection. Use machine guards and take appropriate safety measures to contain dust and to prevent the inhalation of dust. WARNING: This product could expose you to chemicals including cobalt, which are known to the State of California to cause cancer. For more information go to www.P65Warnings.ca.gov. WARNING: Cancer– www.P65Warnings.ca.gov. WARNING: This product contains or produces chemicals which are “known to the state of California to cause cancer.” (Proposition 65) |

3: Composition/Information on ingredients

Substance Name; CAS Number; Concentration range, % by weight; GHS; CLP – Harmonized classification. Tungsten carbide; 12070-12-1; > 60% Cermets grades: 0 – 20%; Not classified. Cobalt Powder (>99% <1mm). (Respirable fraction ≥0,01% w/w); 7440-48-4; 1-25%; Carc 1B, H350, Eye Irrit. 2B, H320 Repr. 1; H360Fd, Muta 2; H341 Acute Tox. 1, H330 Acute Tox. 4, H302 Resp. Sens. 1B, H334 Skin Sens. 1, H317 Aquatic Acute 1 (M=10), H400 Aquatic Chronic 1, (M=1), H410; Carc 1B, H350, Repr. 1; H360F, Resp. Sens. 1, H334, Muta 2; H341, Skin Sens. 1, H317 , Aquatic Chronic 4, H413. Chromium carbide; 12012-35-0; 0-10%; Not classified.

| 4: First aid measures |  |  |
| --- | --- | --- |
| 4.1: Description of first aid measures |  |  |
| Exposure to high volumes of powder/dust is not anticipated under normal conditions and use of sintered hardmetal articles. Operations such as grinding, cutting, burning and welding of hardmetal articles may produce dust or fumes, which can be inhaled, ingested or come in contact with the skin and eyes. |  |  |
| Eyes | Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. If eye irritation persists: Get medical advice/attention. |  |
| Inhalation | If breathing is difficult, remove victim to fresh air and keep at rest in a position comfortable for breathing. If experiencing respiratory symptoms: Call a POISON CENTER or doctor/physician |  |
| Ingestion | Rinse mouth with water (only if the person is conscious). Give plenty of water to drink. Seek medical advice if necessary. |  |
| Skin | Remove contaminated clothing immediately and wash affected skin with plenty of water or soap and water. If skin irritation or rash occurs: Get medical advice/attention. |  |
| General advice | After first aid, get appropriate medical attention. |  |
| 4.2: Most important symptoms and effects, both acute and delayed |  |
| Hard metal dust/mist may cause inflammation of the airways and irritate nose, throat, skin and eyes. In general, metal powders or dust may cause mechanical eye and skin irritation. In workers acute inhalation of powder or dust may cause mild respiratory tract irritation and delayed effects may include occupational asthma and/or skin sensitization. Chronic inhalation of hard metal powder/dust has the potential for causing transient or permanent respiratory disease, including interstitial lung fibrosis. Symptoms include productive cough, wheezing, shortness of breath, chest tightness and weight loss. Certain pulmonary conditions may be aggravated by exposure. In a two-year study on rats and mice, inhalation of cobalt was shown to cause cancer. Cobalt and hardmetal are known sensitizers having potential to cause allergy through repeated exposure. A sensitized person could react with asthmatic symptoms or eczema. |  |
| 4.3: Indication of any immediate medical attention and special treatment needed |  |
| None known. |  |

| 5: Firefighting measures |  |
| --- | --- |
| 5.1: Extinguishing media |  |
| Hard metal articles as provided are not a fire hazard. |  |
| 5.2: Special hazards arising from hardmetal article use |  |
| During normal operation and use, hard metal articles are not a fire hazard. |  |
| 5.3: Advice for firefighters |  |
| Not applicable. |  |

| 6: Accidental release measures |  |
| --- | --- |
| 6.1: Personal precautions, protective equipment and emergency procedures |  |
| Hard metal articles as provided do not present hazards that require accidental release measures. Avoid generation of dust. Do not breath dust or mist. Avoid ingestion and contact with the skin and eyes. Use personal protective equipment (i.e., gloves, safety goggles, dust respirator) as specified in Section 8 of this safety information sheet. Ventilate area if necessary. Hard metal articles may fragment in use. Always wear appropriate safety equipment at all times and ensure all machine guards and safety interlocks are in place prior and during the operation. |  |
| 6.2: Environmental precautions |  |
| In case of generation of dust/mist, avoid release into the environment. |  |
| 6.3: Methods and materials for containment and clean up |  |
| Used tools and broken hard metal components should be recycled. Avoid generation of dust and avoid dispersion of dust in air. Do not shake clothing, rags or other items to remove dust. Dust should be removed by laundering or vacuuming (with appropriate filters). Clean up using methods that avoid dust generation such as a high-efficiency particulate air (“HEPA”) vacuum, wet dust mop or wet cleanup. Place/dispose into appropriate sealable containers. |  |
| 6.4: Reference to other sections |  |
| See sections 8 and 13 for exposure controls and disposal considerations. |  |

| 7: Handling and storage |  |  |
| --- | --- | --- |
| 7.1: Precautions for safe handling |  |  |
| Under normal operating conditions, the use of hard metal articles does not require special safety precautions beyond normal safety procedures, such as safety glasses and gloves. Make sure the drill bits/components are correctly secured in their holder before use, to prevent them coming loose during the process operation. Too much overhang could result in vibration and lead to tool damage/breakage. Always wear appropriate safety equipment at all times and ensure all machine guards and safety interlocks are in place prior and during the operation. DO NOT USE any tool or article that shows signs of damage. Use all appropriate safety guards or machine encapsulations to securely collect particles such as chips or cutting elements that may spin off. Operations such as grinding, cutting, burning, and welding of hard metal articles may produce dust or fumes, which can be inhaled, swallowed or come in contact with the skin and eyes. Hard metal articles should not be reground or sharpened without taking appropriate safety measures to contain dust and to prevent exposure to dust (e.g., ventilation, personal protective equipment). See Section 6.3 for methods for cleanup. Use personal protective equipment (i.e., gloves, safety goggles, dust respirator) as specified in Section 8 of this safety information sheet. Avoid smoking, eating, or drinking while using hard metal articles. Wash hands thoroughly after handling and work shift as well as before eating, drinking, and smoking. |  |  |
| 7.2: Conditions for safe storage, including any incompatibilities |  |  |
| Hard metal articles as provided do not present hazards requiring precautions for safe storage. |  |  |
| 7.3: Specific end use(s) |  |  |
| Hard metal articles are used as cutting and machining tools, mining and drilling tools and wear parts. |  |  |
| 8: Exposure controls/ personal protection |  |
| The exposure control parameters listed below are for operations that generate, or could cause exposure to, dusts or fumes, including grinding, cutting, or re-sharpening. |  |
| 8.1: Control parameters |  |

In addition to limit values for cobalt, national/regional limit values for other substances must be complied with.

| Country | For tungsten and insoluble compounds, as tungsten |  |  | Cobalt |  |
| --- | --- | --- | --- | --- | --- |
|  | 8-h Limit Value (mg/m3) | Short-term Limit Value (mg/m3) | 8-h Limit Value (mg/m3) |  | Short-term Limit Value (mg/m3) |
| Austria | 5 | 10 | 0.1 |  | 0.4 |
| Belgium | 5 | 10 | 0.005** |  | – |
| Canada (Québec) | 5 | 10 | 0.005** |  | – |
| Czech Republic | – | – | 0.05 |  | 0.1† |
| Denmark | 5 | 10 | 0.01 |  | 0.02 |
| Germany | – | – | 0.005#,& 0.0005#,$ |  |  |
| Hungary | – | – | 0.02 |  | – |
| Poland | 5 | – | 0.02 |  | – |
| Spain | 5 | 10 | 0.02 |  | – |
| Sweden | 5 | – | 0.02* |  | – |
| Switzerland | 5 | – | 0.05* |  | – |
| USA (ACGIH TLV) | 3# | - | 0.005** |  | - |
| USA (NIOSH) | 5 | 10† | 0.05 |  | – |
| USA (OSHA) | – | – | 0.1 |  | – |
| UK | 5 | 10 | 0.1 |  | – |

**Thoracic fraction; #Respirable fraction; &Workplace exposure concentration corresponding to the proposed tolerable cancer risk; $Workplace exposure concentration corresponding to the acceptable cancer risk. †NPK-P - highest permissible concentration. Source: GESTIS – International Limit Values for chemical agents (Occupational exposure limits, OELs)

| 8.2: Exposure controls |  |
| --- | --- |
| All personal protective equipment should meet all regional and national requirements. Always wear safety equipment and keep machine guards in place. If tool or article shows signs of damage, return tools to appropriate location for repair, replacement, or recycling. DO NOT use or operate damaged tools or articles. Cutting, sharpening, or grinding hard metal articles and tools produces dust or mist that, at levels above the occupational exposure limit, could cause permanent respiratory disease, and could irritate nose, throat, skin, and eyes. Hard metal articles and tools should not be reground or sharpened without taking appropriate safety measures to contain dust and to prevent the inhalation of dust. Hard metal articles and tools may break or shatter in use. To avoid injury, use personal protective equipment, including eye protection. |  |
| Individual protection measures: |  |
| Eye/face protection | Use safety glasses as appropriate and reasonably necessary. |
| Skin protection | Use work gloves and work clothes as appropriate and reasonably necessary. |
| Respiratory protection | In the case of dust generation, avoid inhalation. Use respiratory protection as appropriate and reasonably necessary. |  |

9: Physical and chemical properties

Not applicable for hard metal articles and tools.

| 10: Stability and reactivity |  |
| --- | --- |
| 10.1: Reactivity |  |
| Hard metal articles are not reactive. |  |
| 10.2: Chemical stability |  |
| Hard metal articles are chemically stable. |  |
| 10.3: Possibility of hazardous reactions |  |
| Not applicable. |  |
| 10.4: Conditions to avoid |  |
| Cutting, sharpening, or grinding hard metal articles and tools produces dust or mist that, could cause permanent respiratory disease, and could irritate nose, throat, skin, and eyes. Hard metal articles and tools should not be reground or sharpened without taking appropriate safety measures to contain dust and to prevent the inhalation of dust. Hard metal articles and tools may break or shatter in use. To avoid injury, use personal protective equipment, including eye protection. |  |
| 10.5: Incompatible materials |  |
| None known. |  |
| 10.6: Hazardous decomposition products |  |
| None known. |  |
| 11: Toxicological information. Under normal conditions of use, hard metal articles do not present a human health hazard. However, hard metal dusts or fumes contain hazardous substances that may be inhaled or ingested or come into contact with the skin or eyes. Routes of Likely Exposure to Hard metal: The most relevant routes of potential exposure to workers would be the dermal (skin) and inhalation routes. Symptoms Related to the Physical, Chemical and Toxicological Characteristics: In general, hard metal dust may cause mechanical eye and skin irritation. Inhalation of dust may cause mild respiratory tract irritation. Cobalt ¾Tungsten Carbide: Cobalt metal with tungsten carbide was categorized by IARC as probably carcinogenic to humans (Group 2A). The US NTP considers cobalt-tungsten carbide (powders and hard metals) as reasonably anticipated to be a human carcinogen. STOT- Repeated Exposure: Chronic inhalation has the potential for causing transient or permanent respiratory disease, including occupational asthma and interstitial fibrosis. It is reported that cobalt dust is the most probable cause of such respiratory diseases. Symptoms include productive cough, wheezing, shortness of breath, chest tightness and weight loss. Interstitial fibrosis (lung scarring) could lead to permanent disability. Certain pulmonary conditions may be aggravated by exposure. Tungsten Carbide: Dust may cause mechanical irritation to the eyes, skin, and respiratory tract. One source with additional information is: https://wwwn.cdc.gov/TSP/PHS/PHS.aspx?phsid=804&toxid=157 Cobalt: Cobalt is classified as May cause cancer. The IARC lists cobalt metal with tungsten carbide as Category 2A – probably carcinogenic to humans. Cobalt fumes or dusts may cause pulmonary, skin or eye irritation. Cobalt is a sensitizing agent for the skin and respiratory system. Cobalt is classified as fatal if inhaled, and harmful if swallowed. Cobalt is classified as Suspected of causing genetic defects. Cobalt is classified as May damage fertility and suspected of damaging the unborn child. There are reports that chronic exposure could affect the heart, pancreas, thyroid gland, or bone marrow. One source with additional information regarding the hazards associated with cobalt metal powder is: https://wwwn.cdc.gov/TSP/PHS/PHS.aspx?phsid=371&toxid=64. 12: Ecological information. Hard metal articles as provided do not present an environmental hazard.. 12.1: Persistence and degradability. Not applicable.. 12.2: Bio accumulative potential. Not applicable.. 12.3: Mobility in soil. Not applicable.. 12.4: Results of PBT and vPvB assessment. Tungsten carbide and cobalt are inorganic substances, and therefore the PBT and vPvB assessment is not required.. 12.5: Other adverse effects. None known.. |

13: Disposal considerations

Responsibility for proper waste disposal of hard metal articles is with the owner. Owners are encouraged to take advantage of carbide recycling programs. Hard metal articles are valuable articles that should be sent to an appropriate reclamation facility. If material cannot be sent to a reclamation facility, dispose of all waste products and containers in accordance with local, state/provincial, federal, and national regulations.

14: Transport information

Hard metal articles are not classified or regulated.

15: Regulatory information

15.1: Safety, health and environmental regulations/legislation specific for the article

EU Regulations: Hard metal articles do not contain substances of very high concern (“SVHC”) above 0.1% National Regulations (USA): # Occupational Safety and Health Act (OSHA): Federal OSHA Hazard Communication Standard 29 CFR 1910.1200. Toxic Substances Control Act (“TSCA”): Components of this article are listed on the TSCA inventory. # Superfund Amendments and Reauthorization Act (“SARA”): Cobalt is subject to the requirements of Section 313 of Title III of the Superfund Amendment and Reauthorization Act of 1986. # State Regulatory Information: This article contains cobalt, which is listed in California Proposition 65 as known cancer-causing chemicals. # Other applicable state/regional/national regulations shall be followed when applicable.

15.2: Chemical safety assessment - EU

Not applicable for Hard metal articles.

| 16: Other information |  |  |  |
| --- | --- | --- | --- |
| Full text of classifications (CLP/GHS) | Acute Tox. 1, H330 Acute Tox. 4, H302 Aquatic Acute 1 (M=10), H400 Aquatic Chronic 1, (M=1), H410 Aquatic Chronic 3, H412 Aquatic Chronic 4, H413 Carc 1B, H350 Carc. 2, H351 Eye Irrit. 2B, H320 Muta 2; H341 Repr. 1; H360Fd Repr. 1, H360F Resp. Sens. 1, H334 Resp. Sens. 1B, H334 Skin Sens. 1, H317 STOT RE 1, H372 |  | Acute toxicity, category 1 Acute toxicity, category 4 Aquatic toxicity (Acute), category 1, M-factor 10 Aquatic toxicity (Chronic), category 1, M-factor 1 Aquatic toxicity (Chronic), category 3 Aquatic toxicity (Chronic), category 4 Carcinogenic, category 1B Carcinogenic, category 2 Eye irritation, category 2B Mutagenic, category 2 Reproductive toxicity, category 1 Reproductive toxicity, category 1 Respiratory sensitization, category 1 Respiratory sensitization, category 1B Skin sensitization, category 1 Specific target organ toxicity - repeated exposure, category 1 |
| Full text of abbreviated H statements | H302 H317 H320 H330 H334 H341 H350 H351 H360Fd H360F H372 H400 H410 H412 H413 | Harmful if swallowed May cause an allergic skin reaction Causes eye irritation Fatal if inhaled May cause allergy or asthma symptoms or breathing difficulties if inhaled Suspected of causing genetic defects May cause cancer Suspected of causing cancer May damage fertility. Suspected of damaging the unborn child May damage fertility Causes damage to organs through prolonged or repeated exposure Very toxic to aquatic life Very toxic to aquatic life with long lasting effects Harmful to aquatic life with long lasting effects May cause long lasting harmful effects to aquatic life |  |
| Revision(s): | Safety information data sheet prepared on 9 November 2022 Note in particular classification changes and occupational exposure limits. |  |  |
| Abbreviations: |  |  |  |  |
| ACGIH; American Conference of Industrial Hygienists. CAS; Chemical Abstracts Service. Cat; Category. CLP; Classification, Labelling and Packaging. EINECS; European Inventory of Existing Commercial Chemical Substances. EU; European Union. GHS; Globally Harmonized System. h; Hours. IARC; International Agency for Research on Cancer. m3; Cubic meter. mg; Milligram(s). NIOSH; National Institute for Occupational Safety and Health. No.; Number. NTP; National Toxicology Program. OEL OSHA; Occupational Exposure Level Occupational Safety and Health Administration. PBT; Persistent, Bio accumulative, and Toxic. SVHC; Substance of Very High Concern. TLV vPvB; Threshold Limit Values very Persistent, very Bio accumulative. |

# User Responsibilities

This SIS provides information consistent with recommended applications of these products/articles and anticipated activities involving the products/articles. It is the user's responsibility to identify and protect against health and safety hazards presented by modification of hard metal products/articles after manufacture.  Individuals handling hard metal should be informed of all relevant hazards and recommended safety precautions and should have access to the information contained in this SIS.

# Disclaimer

The information contained herein is based upon data provided by manufacturers and suppliers of raw materials used in the manufacture of hard metal powders and products/articles.  The information is offered in good faith as accurate and correct, but no representations, guarantees, or warranties of any kind are made as to its accuracy or completeness, suitability for particular applications, hazards connected with the use of the products/articles, or the results to be obtained from the use thereof.  User assumes all risk and liability of any use or handling of any material beyond Sandvik’s control.  Variations in methods; conditions; equipment used to store, handle, or process the material; and hazards connected with the use of the hard metal products/articles are solely the responsibility of the user and remain at the user’s sole discretion.

This SIS is not intended to preempt, replace, or expand the terms contained in the Sandvik Conditions of Sale. Compliance with all applicable federal, state, and local laws and regulations remains the responsibility of the user, and the user has the responsibility to provide a safe workplace, to examine all aspects of its operation, and to determine if or where precautions, in addition to those described herein, are required.  This information may not be valid for hard metal products/articles when manufactured with alternate materials meeting the special requirements of a particular user.

Sandvik makes no warranties, expressed or implied, including, but not limited to, the implied warranties of merchantability and fitness for a particular purpose.

End of Safety Information Sheet